Loco Repair Electrical Section

In Charge: Hafizur Rahman Milon (SAE)

Problem:

- Manpower

- Just x1 single SAE Incharge

Functions:

- Receive and Make DD list of locomotives

- Disassemble and assemble electrical components of locomotives

- Locomotive primary cleaning and painting

- Electrical component fitting

- HV and LV wiring of locomotives

- Different electrical circuit modifications

- Locomotive fuel tracker setting

- Locomotive vigilance system setting

- Engine start and check

- Load Box test of locomotives

- Light and loaded trial run of locomotives (check for electrical faults)

- Handover to operating divisions (focus on electrical performance)

Before Disassemble:

1. DD List Prepare

During Disassembly:

2. Electrical Compoents disassembly and sending to relevant sections

3. Chassis Cleaning

After Disassembly:

4. Hight Voltage (HV >74V) & Low Voltage (LV <=74V) cable inspection.

- example of HV cable line: main generator - electricala cabinet - (reverser + power conductor) - traction motor

- all other lines: LV line

Reassembly:

5. Assemble and wiring

6. Engine Start

7. Troubleshooting

8. Load Box Testing

9. Light+Loaded Trial Run

10. Handover

Currently ongoing projects:

- 2313 handover processing

- 2708 disassembling done