# Truck and Bogie Repair Section

In Charge: Miss Soma Biswas SSAE

---

Problems:

1. permanent staff needed according to organogram: 12-13 persons. currently have: 6 persons (ie, manpower shortage)

x2 fitters

x2 khalasi

x2 (1 female and 1 handicapped)

Functions:

- Repair of truck and bogie of locomotives

- Traction motor fitting and rotation test

- Gear case

- Truck frame tramming

- Centre buffer coupling

- Brake cylinder

Major Machines and Parts:

- Spring Testing Machine

Achievements:

- Gear case repair fixtures

- Modification of CBC coupler of BEB-22 locomotives

functions and main works (as stated by the incharge):

1. cut motor connection

2. bring out the truck out of locomotives

3. separate the motors from the truck

4. separate the traction motor

5. tdisassembly of traction motor, and then send the motor to EM (electrical machines) section

6. axle box disassembly (and send to wheel section)

7. bearing

8. truck frame clearing and disassembling

9. jaw buildup

10. tramming of truck frame

11. reversing the truck frame to work on it.

12. brake cylinder disassemble, clean, repair and assemble

13. brake head repair

14. hanger liver repair

15. cap padestal repair

16. axle box repair

17. finally assemble of track frame

Some pictures: