Electrical Machines Section

InCharge:: Masud Khan (SSAE)

major 2 things

- traction motor

- main alternator/generator

other important components:

- cooling fan,

- water pump,

- starting motor,

- blower motor

- generator axle clutch,

- Eddy Current clutch etc.

Problem: stock of components not good of traction motors

Frequent defect:

Motors are grounded

HiPot Test (high potential) testing is done in this section… which is done when the components are cleaned and dried to the fullest. Approximately 2000 volts are applied during the test.

Major reason behind the motors getting grounded:

In divisions (CTG, DHK, LMH, PXC), in the locosheds, during maintenance, they have little time to clean properly… darts, waters etc get infiltrated inside the motors, thus, when running the loco, the coils get short circuited or otherwise grounded.

Another problem of frequent grounding of electrical machines in locomotives, is that the trial run after GOH is not sufficiently long in period so as to ascertain that the electrical machines parts are properly overhauled.

- Another point is that the stators of the motors and/or generators do not get defected much often.

- The armatures are gone through rewinding by outsourcing (to the contractors)… we cannot do that due to shortage of manpower.

Another great problem that the incoming locomotives have:

Bearings are getting seized regularly. Mostly because the bearing parts are not sufficiently qualified.

Major parts of alternators/generators that the section is concerned with:

- Main pole

- Interpole

- armature

manpower shortage scenario:

56 SSAE (Senior Sub Assistant Engineer) present at 2006 in all the sections of CLW, in 2006

Only 10 pcs of SSAE currently posted here in CLW (all sections combined)

This Electrical Machines section had 32 persons permanently employed in 2006, currently have only 8 persons.

knowledge transfer problem of staff: the experienced Mistris (Fitters) are gone to PRL (Post Retirement Leave), but the later manpower shortage problem results in less skilled employees… those experienced (who have retired) could not transfer the knowledge to juniors properly.