Mechanical Reconditioning Section

In Charge: Pronay Kumar Das (SSAE)

---

Major Components that this section deals with:

- Compressor

- Turbocharger

- piston & connecting rod

- liner

- cross head lifter (alco engines)

- rocker arm

- cylinder head

- traction motor blower (both GM and ALCO engines)

- AC blower (ie. Aftercooler Core blower) (for ALCO engines)

- water pump

- lube oil pump

- crankcase pressure detector (for GM engines)

- relief valve regulating valve

- relief valve bypass valve

- governor drive gear

- lube oil cooler

- cooling fan

- lube oil separator

- soakback pump

- thermostat valve

- low water cutoff valve

- eddy current clutch (ALCO engines)

## Compressor

ALCO type compressor: 6CD-4UC... ie. 6 cylinder, 4  exhauster

x4 exhauster

x2 compressors

the compressor unit has breather valve

air flow:

atmostphere -> low pressure compressor cylinder -> intercooler -> high pressure compressor cylinder -> main reservoir