Aftercooler Core Blower / AC Blower (ALCO) (Easy to understand, yet complete)

1. What is the Aftercooler Core Blower (AC Blower)?

The Aftercooler Core Blower (often called AC Blower) is a fan used mainly on ALCO locomotives. Its job is to force cooling air through the aftercooler (also called intercooler or charge-air cooler).

2. Why is it needed?

Remember what the turbocharger does:

- It compresses air and pushes it into the engine.

- Compressing air makes it hot.

Hot air contains less oxygen. So the compressed air must be cooled before it enters the engine cylinders. This cooling is done by the aftercooler.

The AC Blower’s only job is to push plenty of cool outside air across the aftercooler core so that the hot compressed air inside the core loses its heat.

3. How it works (simple flow)

- Turbocharger compresses fresh air → air becomes hot.

- Hot compressed air flows through the tubes of the aftercooler.

- The AC Blower forces cool outside air across those tubes.

- Heat is transferred from the hot air inside the tubes to the cool air outside.

- The now-cooled (and denser) air goes into the engine cylinders.

Result: More oxygen reaches the cylinders → better combustion → more power and less smoke.

4. Important points

| Point | Explanation |
| --- | --- |
| Used mainly on | ALCO locomotives |
| What it cools | Compressed air coming from the turbocharger |
| Drive | Usually electric motor or mechanically driven |
| Location | Near the aftercooler core |
| If it fails | Compressed air remains hot → loss of power, more smoke, higher exhaust temperature |

5. Difference from Traction Motor Blower

- Traction Motor Blower → cools the electric motors that drive the wheels.

- AC Blower → cools the compressed air going into the diesel engine.

They are two completely different cooling fans with different purposes.

6. Simple one-sentence summary

The Aftercooler Core Blower (AC Blower) on ALCO locomotives forces cooling air through the aftercooler so that the hot compressed air from the turbocharger is cooled before entering the engine.