Cylinder Head (Easy to understand, yet complete)

1. What is a Cylinder Head?

The cylinder head is the heavy, strong metal cover that sits on top of the engine block and seals the top of each cylinder. It is one of the most important parts of the engine.

2. Why is it needed?

The cylinder head does several critical jobs:

- It seals the combustion chamber so the high-pressure burning gases cannot escape.

- It holds the inlet and exhaust valves.

- It holds the fuel injector.

- It provides passages for air to enter and exhaust gases to leave.

- It has passages for cooling water and lubricating oil.

- It supports the rocker arms and valve mechanism.

Without a strong and properly sealed cylinder head, the engine cannot produce power.

3. Main parts found on / in the cylinder head

| Part | Function (simple) |
| --- | --- |
| Valves (Inlet & Exhaust) | Control the flow of air in and exhaust gases out |
| Valve seats | Hard rings on which the valves sit when closed |
| Valve guides | Guide the valve stems so they move straight |
| Fuel injector | Sprays diesel fuel into the cylinder |
| Rocker arms & shaft | Open the valves |
| Cooling water passages | Allow coolant to remove heat from the head |
| Oil passages | Supply lubricating oil to the valve gear |
| Head gasket surface | Seals against the engine block and liner |

4. How it works with other parts

- The piston moves up and compresses air inside the liner.

- The cylinder head seals the top of this space, forming the combustion chamber.

- The injector sprays fuel into this chamber.

- The fuel burns, creating high pressure that pushes the piston down.

- The valves in the cylinder head open and close at the right time to let air in and exhaust out.

5. Cylinder Head Gasket / Joint

A special gasket (or in some engines a metal-to-metal joint with seals) is placed between the cylinder head and the engine block/liner. This joint must be perfect. If it leaks:

- Combustion gases can escape

- Coolant can enter the cylinder

- Oil and water can mix

That is why the cylinder head bolts are tightened in a very specific sequence and to exact torque values during assembly.

6. ALCO vs GM

Both ALCO and GM engines have cylinder heads, but:

- The shape, number of valves, injector mounting, and cooling arrangement are different.

- Some designs have individual heads for each cylinder; others have multi-cylinder heads.

- The method of sealing and the valve arrangement can vary.

7. What can go wrong?

- Cracked cylinder head (due to overheating)

- Warped sealing surface

- Burnt or leaking valves

- Damaged valve seats

- Coolant or oil leakage

- Improper torque of head bolts leading to gasket failure

All of these problems cause loss of power, overheating, or serious engine damage.

8. Simple one-sentence summary

The cylinder head is the strong cover on top of the cylinders that seals the combustion chamber, holds the valves and injector, and allows air, exhaust, coolant, and oil to flow correctly.

Next in the list: Traction motor blower (both GM and ALCO)

Say “next” when you are ready