Cylinder Liner (also called Cylinder Sleeve) (Easy to understand, yet complete)

1. What is a cylinder liner and why is it used?

The cylinder liner is a hard, replaceable metal tube (sleeve) that fits inside the engine block.

The piston moves up and down inside this liner, not directly inside the engine block.

Why use a separate liner?

- The liner can be replaced when it becomes worn or damaged.

- It is made of a special hard material that resists wear and heat much better than the engine block itself.

- It makes major overhauls easier and cheaper.

2. How it works

- The liner is inserted into the engine block (it is a precise fit).

- The piston (with its rings) slides up and down inside the smooth inner surface of the liner.

- Combustion happens on top of the piston, inside the liner.

- Cooling water flows around the outside of the liner (in the water jacket) to remove heat.

- Lubricating oil coats the inner wall so the piston rings can slide smoothly.

3. Important features of a locomotive cylinder liner

| Feature | Purpose (simple) |
| --- | --- |
| Hard inner surface | Resists wear from the piston rings |
| Precision bore | Must be perfectly round and smooth for proper sealing |
| Cooling passages | Allow water to flow around the outside to keep it cool |
| Sealing rings / O-rings | Prevent cooling water and oil from mixing |
| Flange or shoulder | Holds the liner in the correct position in the block |

4. Wet liner vs Dry liner

Most large diesel locomotive engines (both ALCO and GM) use wet liners:

- Wet liner: The outer surface of the liner is directly in contact with the cooling water. This gives excellent cooling.

- Dry liner: The liner does not touch the water; it is pressed into a dry bore. (Less common in large locomotive engines.)

5. What happens when the liner wears out?

- The inner surface becomes scored, oval, or worn.

- Piston rings can no longer seal properly.

- Result: loss of power, high oil consumption, blow-by gases, and black or blue smoke.

- In severe cases, the liner can crack.

During overhaul, technicians measure the liner bore carefully. If it is beyond limits, the liner is replaced.

6. ALCO vs GM (simple note)

Both ALCO and GM engines use replaceable cylinder liners. The exact dimensions, material, and sealing arrangements are different, but the basic idea and function are the same.

7. Simple one-sentence summary

The cylinder liner is a hard, replaceable metal sleeve inside which the piston moves; it provides a durable wearing surface and allows the engine to be overhauled without replacing the entire engine block.