Lube Oil Separator (Easy to understand, yet complete)

1. What is the Lube Oil Separator?

The lube oil separator (also called crankcase breather separator or oil mist separator) is a device that removes oil droplets and oil mist from the air that comes out of the engine crankcase.

2. Why is it needed?

Inside a running engine, some oil is always thrown around as fine mist or small droplets. This oil mist mixes with the air (blow-by gases) that is vented from the crankcase.

If this oily air is released directly into the atmosphere:

- It creates pollution

- Oil is wasted

- The engine room or area around the locomotive becomes oily and dirty

- In some cases it can create a fire risk

The separator cleans this air by removing the oil before the air is discharged.

3. How it works (simple)

- Air mixed with oil mist leaves the crankcase.

- It enters the lube oil separator.

- Inside the separator, the air is forced to change direction many times or pass through special baffles/filters.

- The heavier oil droplets hit the walls or filter media and stick there.

- The separated oil drains back into the engine oil sump.

- The cleaner air is then released to the atmosphere (or sometimes re-circulated).

4. Important points

| Point | Explanation |
| --- | --- |
| Purpose | Removes oil mist from crankcase ventilation air |
| Oil recovery | Separated oil returns to the sump (saves oil) |
| Cleaner exhaust air | Reduces pollution and oil mess around the locomotive |
| Maintenance | Needs periodic cleaning; can become clogged with sludge |
| If blocked | Crankcase pressure can rise abnormally |

5. Relation to other systems

- Works together with the crankcase ventilation system.

- On GM engines it is especially important because they also have a Crankcase Pressure Detector.

- Helps keep the engine room cleaner and reduces oil consumption slightly.

6. ALCO vs GM

Both ALCO and GM locomotives are fitted with some form of lube oil separator / crankcase breather system. The basic purpose is the same; the physical design and location may differ.

7. Simple one-sentence summary

The lube oil separator removes oil droplets from the air that is vented from the crankcase, returns the oil to the sump, and releases cleaner air to the atmosphere.