Relief Valve / Regulating Valve (Easy to understand, yet complete)

1. What is a Relief Valve / Regulating Valve?

A relief valve (also called regulating valve) is a safety and control valve that automatically opens when the pressure in a system becomes too high. It protects the system by releasing the excess pressure.

In diesel locomotives, the most common and important one is the lube oil pressure regulating (relief) valve.

2. Why is it needed?

The lube oil pump is driven by the engine. As engine speed increases, the oil pump can produce higher and higher pressure.

If this pressure is not controlled:

- Oil seals can be damaged

- Oil filters can burst

- Hoses and pipes can leak or rupture

- Excess oil can be forced into places where it should not go

The relief / regulating valve keeps the oil pressure within a safe operating range.

3. How it works (simple)

- Oil from the lube oil pump flows toward the engine.

- A spring-loaded valve is set to a specific pressure (for example 4–6 bar, depending on the engine).

- As long as the oil pressure is below the set limit, the valve stays closed and all oil goes to the engine.

- When the pressure tries to rise above the set limit, the valve opens.

- Excess oil is allowed to return directly to the oil sump (or to the suction side of the pump).

- As soon as the pressure drops back to normal, the spring closes the valve again.

This happens automatically and continuously.

4. Important points

| Point | Explanation |
| --- | --- |
| Main purpose | Prevents oil pressure from becoming dangerously high |
| Location | Usually on or near the lube oil pump or oil gallery |
| Adjustment | Set to a specific pressure during maintenance |
| If it sticks closed | Oil pressure can become too high → damage to seals/filters |
| If it sticks open | Oil pressure stays too low → risk of bearing damage |

5. Other relief valves in the locomotive

Similar relief valves are also used in:

- Fuel system

- Air compressor system

- Cooling system (in some designs)

- Hydraulic systems (if fitted)

But the most critical one for the engine itself is the lube oil regulating/relief valve.

6. ALCO vs GM

Both ALCO and GM engines use oil pressure regulating/relief valves. The working principle is the same; only the physical design, setting pressure, and exact location differ.

7. Simple one-sentence summary

The relief (regulating) valve automatically opens when oil pressure becomes too high and sends the excess oil back to the sump, keeping the pressure within safe limits.