Rocker Arm (Easy to understand, yet complete)

1. What is a Rocker Arm?

The rocker arm is a small but very important lever that sits on top of the cylinder head. Its job is to open and close the valves at exactly the right time.

2. Why is it needed?

In a diesel engine, each cylinder has valves:

- Inlet valve – lets fresh air in

- Exhaust valve – lets burnt gases out

These valves must open and close at precise moments. The rocker arm is the part that actually pushes the valves open.

3. How it works (step-by-step)

- The engine has a camshaft with egg-shaped lobes (cams).

- As the camshaft rotates, a lobe pushes a pushrod (or directly acts in some designs) upward.

- The pushrod pushes one end of the rocker arm.

- The rocker arm pivots (rocks) on a shaft or stud.

- The other end of the rocker arm pushes down on the valve stem → the valve opens.

- When the cam lobe moves away, a strong spring closes the valve again.

This happens many times every second while the engine is running.

4. Important parts related to the rocker arm

| Part | Function (simple) |
| --- | --- |
| Rocker arm | Lever that opens the valve |
| Rocker shaft | The pivot on which the rocker arm rocks |
| Pushrod | Transfers motion from the camshaft upward |
| Valve spring | Closes the valve when the rocker arm releases it |
| Adjusting screw / Tappet | Used to set the correct valve clearance |

5. Valve clearance (very important)

There must be a tiny gap (clearance) between the rocker arm and the valve stem when the valve is closed. This clearance is carefully adjusted during maintenance. Too little or too much clearance can cause noise, loss of power, or valve damage.

6. ALCO vs GM

Both ALCO and GM engines use rocker arms. The exact shape, size, and method of adjustment may differ, but the basic function is the same.

7. What can go wrong?

- Worn rocker arm or shaft → noisy operation, poor valve timing

- Broken rocker arm → that cylinder stops working properly

- Incorrect valve clearance → loss of power, overheating, or valve burning

8. Simple one-sentence summary

The rocker arm is a pivoting lever on the cylinder head that transfers motion from the camshaft to open the inlet and exhaust valves at the correct time.