Thermostat Valve (Easy to understand, yet complete)

1. What is the Thermostat Valve?

The thermostat valve is a temperature-controlled valve in the engine cooling system. It automatically decides whether the coolant should stay circulating only inside the engine or should go to the radiators for cooling.

2. Why is it needed?

A diesel engine needs to reach its normal operating temperature quickly and then stay there.

- If the engine stays too cold → poor combustion, higher wear, more fuel consumption, and sludge formation.

- If the engine becomes too hot → risk of serious damage.

The thermostat helps the engine warm up fast and then keeps the temperature in the correct range.

3. How it works (simple)

- When the engine is cold

  - The thermostat stays closed.

  - Coolant circulates only inside the engine (short circuit / bypass).

  - The engine warms up quickly.

- When the engine reaches normal temperature

  - The thermostat begins to open.

  - Coolant is allowed to flow to the radiators.

  - Heat is removed by the cooling fans and radiators.

- If the engine gets hotter

  - The thermostat opens further.

  - More coolant flows to the radiators for stronger cooling.

The thermostat continuously adjusts its opening according to the coolant temperature.

4. Important points

| Point | Explanation |
| --- | --- |
| Location | In the coolant circuit, usually at the engine outlet |
| Sensing | Contains a temperature-sensitive element (wax or similar) |
| Cold engine | Stays closed → fast warm-up |
| Hot engine | Opens → coolant goes to radiators |
| Failure (stuck closed) | Engine overheats |
| Failure (stuck open) | Engine takes too long to warm up, runs cooler than ideal |

5. Relation to other parts

- Works together with the water pump, radiators, and cooling fans.

- Helps the low water cut-off and overall cooling system protect the engine.

- On both ALCO and GM locomotives the principle is the same.

6. Simple one-sentence summary

The thermostat valve automatically controls the flow of coolant to the radiators so that the engine warms up quickly and then stays at the correct operating temperature.